Product Specifications

2.5G 8-Port Gigabit 4-Port 10 Gigabit L3 Managed Commercial Switch(SR-S25G3412F)

Product Overview

The SR-S25G3412F series is an L3 managed switch equipped with 8 x 2.5G RJ45 ports, 4 x 10G optical ports, and 1 x Console port. Designed to meet the networking needs of growing enterprises, it features a comprehensive suite of management and security functions. These cost-effective devices provide an ideal solution for various network scenarios in small and medium-sized businesses.

The SR-S25G3412F supports comprehensive Layer 3 management capabilities, including SNMP, ACL, IPv6, LACP load balancing, static routing, 802.1Q VLAN, port isolation, port mirroring, QoS, and storm control. The system employs a new hardware and software platform, delivering innovative technical features and solutions for secure access policies, multi-service support, and simplified management and maintenance. It provides high throughput for the most demanding network edge workgroups while serving as a backbone Gigabit switch and high-speed server backbone in small networks.

Product Features

| Features 8 auto-negotiating 10/100/1000/2500M RJ45 ports and 4 10 Gigabit SFP+ ports. |  |
| --- | --- |
| Compliant with IEEE 802.3/802.3u/802.3ab/802.3bz/802.3x/802.3cb standards |  |
| Full-duplex flow control complies with the IEEE 802.3x standard, while half-duplex employs the Backpressure standard. |  |
| Implement a two-tier user management approach to enhance the security of switch administration. |  |
| The port supports auto-negotiation, automatically adjusting the transmission mode and speed. |  |
| Support | MAC address auto-learning, auto-aging, and aging time configuration; |
|  | Static MAC address, MAC address filtering, dynamic MAC binding, and other address table management functions |
|  | IEEE 802.1Q-based Tag VLAN; |
|  | 16K MAC address; |
|  | Port security control, broadcast storm suppression, and other functions; |
|  | Port aggregation functionality; |
|  | Port mirroring and traffic statistics functionality; |
|  | Port-based/802.1p/DSCP priority QoS. |

Technical Specifications

| Standard | IEEE 802.3/802.3u/802.3ab/802.3bz/802.3z/802.3cb/ |
| --- | --- |
| Port | 8 x 10M/100M/1000M/2.5G RJ45 ports, 4 x 10 Gigabit fiber ports, 1 x Console port |
| Indicator light | 8 link/act indicators, 8 2.5G rate indicators, 4 10G link/act indicators, power input indicator, system light |
| Network medium | 10Base-T: Category 3, 4, or 5 unshielded twisted pair (UTP) cable (maximum 100 meters) 100Base-Tx: UTP category 5, 5e cable (maximum 100 meters) 1000Base-T: UTP category 5e, 6 cable (maximum 100 meters) 2.5GBase-T:UTP category 5e, 6 cable (maximum 100 meters) SFP+:optical fiber |
| Power Input | AC100~240V 50/60Hz |
| Size | 271*181.5*44.5mm |
| Conversion Capacity | 120Gbps |
| Forwarding Rate | 89.28Mpps |
| MAC Address | 16K |
| Cache | 12Mbit |
| Giant Frame | 12KB |
| Environment | Operating temperature: 0°C to 45°C (32°F to 113°F) |
|  | Storage temperature: -40°C to 70°C (-40°F to 158°F) |
|  | Operating Humidity: 10% to 90% RH, non-condensing |
|  | Storage humidity: 5% to 95% RH, non-condensing |

Software Specifications

| VLAN Characteristics | • Supports IEEE802.1Q VLAN |
| --- | --- |
|  | • Supports private VLAN |
|  | • Supports Protocol VLAN |
|  | • Supports flexible QinQ |
|  | • Supports VLAN Translation |
|  | • Supports N:1 VLAN Translation |
|  | • Supports Voice VLAN |
|  | • Supports MAC VLAN |
| DHCP | • Supports DHCP Snooping、DHCP Relay |
|  | • Supports DHCP client |
|  | • Supports Option 82 |
| Reliability | • Supports STP，RSTP，MSTP |
|  | • Supports multiple processes MSTP |
|  | • Supports LACP load balancing |
|  | • Supports the G.8032 standard ring network protocol |
|  | • Supports VCT、DDM |
|  | • Supports ULDP（Implement the same functionality as Cisco UDLD） |
| Multicast | • Support IGMP v1/v2/v3 snooping、IGMP Fast leave |
|  | • Support MLD v1/v2 snooping |
| Service Quality (QoS) | • Supports flow classification based on ACL flows, VLAN ID, COS, TOS, DSCP, and IPv6 FlowLabel |
|  | • Supports port-based and VLAN-based PolicyMap binding |
|  | • Supports DSCP, COS/802.1p priority, TOS, and other field rewriting |
|  | • Supports 8 queues per port |
|  | • Supports SP, WRR, SWRR, and DWRR queue scheduling algorithms |
| Access Control List (ACL) | • Supports IP ACLs, matching based on fields such as source/destination IP, IP protocol type, IP priority, and TCP/UDP source and destination port numbers. |
|  | • Supports customizable access control lists (ACLs) |
|  | • Supports port-based and VLAN-based ACL binding |
|  | • Supports MAC ACLs, enabling packet matching based on source/destination MAC addresses, VLAN IDs, COS, and other fields. |
| routing | • Supports IPv4/IPv6 Gateway |
| Safety | • Support port rate limiting |
|  | • Supports broadcast storm control based on packet and byte counts |
|  | • Supports port/MAC binding and MAC filtering |
|  | • Support for port-based and VLAN-based MAC address limiting |
|  | • Supports ARP spoofing prevention |
|  | • Supports DHCP Snooping、DHCP Relay |
|  | • Supports DAI |
|  | • Supports standard IEEE 802.1x and extended IEEE 802.1x |
|  | • Supports TACACS+ |
| Management and Operations | • Supports CLI |
|  | • Supports Ethernet management port |
|  | • Supports IPv4/IPv6 HTT |
|  | • Supports IPv4/IPv6 SNMP |
|  | • Supports RADIUS authentication for Telnet usernames and passwords over IPv4/IPv6 |
|  | • SSH supporting IPv4/IPv6 |
|  | • Supports syslog |
|  | • Supports SNMPv1/v2c/v3 |
|  | • Supports MIB interface, supports Trap |
|  | • Supports dual IMG and multiple configuration files |
|  | • Supports LLDP |
|  | • Supports port mirroring |
|  | • Supports one-click reset |
|  | • Supports four sets of RMON: 1, 2, 3, and 9 |